ASSESSMENT 36

| Date: | 29-06-2020 | Name: | Sheela Golasangi |
| --- | --- | --- | --- |
| Course: | IIRS Outreach Program on Satellite Photogrammetry | USN: | 4AL16EC068 |
| Topic: | Introducing Photogrammetric Concepts | Semester & Section: | VIII ‘B’ |
| Github Repository: | Sheela-Course |  |  |

Report: Map: It shows an area as seen vertically form above. Different symbols and colours are used to represent various objects on a map. Aerial Photo: They are taken from an aircraft to show objects on the ground. They can be divided into vertical aerial photos and oblique aerial photos. WHAT IS PHOTOGRAMMETRY • The science of quantitative analysis of measurements from photographs • Photos - light • Gramma - to draw • Metron - to measure Distinct Areas in Photogrammetry Metric Photogrammetry • Making precise measurements from photos determine the relative locations of points. • Finding distances, angles, areas, olumes, elevations, and sizes and shapes of objects. Interpretative Photogrammetry • Deals in recognizing and identifying objects and judging their significance through careful and systematic analysis. Most common applications: • preparation of planimetric and Interpretation Sensing topographic maps • production of digital orthophotos • Military intelligence such as targeting BRANCHES OF PHOTOGRAMMETRY Based on platform: • Ground Based • UAV/drone based • Aerial Photogrammetry • Satellite Photogrammetry Based on processing techniques: Analogue System - Optical or mechanical instruments were used to reconstruct three-dimensional geometry from two overlapping photographs - The main product during this phase was topographic maps Digital System - Digital photogrammetry is applied to digital images that are stored and processed on a computer - Digital photogrammetry is sometimes called softcopy photogrammetry. - The output products are in digital form, such as digital maps, DEMs, and digital orthophotos saved on computer storage media. Analytic system - The computer replaces some expensive optical and mechanical components - Devices were analog/digital hybrids - Main developments- Analytical aerotriangulation, analytical plotters, and orthophoto projectors - Outputs - can be topographic maps, but can also be digital products such as digital maps and DEMs BASIC CONCEPT - The primary objective of the technique is to derive precise coordinates of a point - This is done by viewing the area from two different angles, thereby recreating the same conditions as it existed at the time of photography. TYPES OF AERIAL PHOTOGRAPHY - Vertical - Low oblique - High oblique Scale of Aerial Photography Before a photograph can be used as a map supplement or substitute, it is necessary to know its scale. On a map, the scale is printed as a representative fraction that expresses the ratio of map distance to ground distance, For example: RF=MD/ GD On a photograph, the scale is also expressed as a ratio, but is the ratio of the photo distance (PD) to ground distance. For example: RF PD/GD scale = f / H scale = photo distance + ground distance